IOP-OSPMC-125217901

Multi Crystalline Solar Modules

125W / 21.60V / 7.91A

| Specification |  |
| --- | --- |
| Maximum Power Rating (W) ± 3% | 125W |
| Open Circuit Voltage (Voc) | 21.60V |
| Short Circuit Current (Isc) | 7.91A |
| Maximum Power Voltage (Vmp) | 16.96V |
| Maximum Power Current (Imp) | 7.39A |
| Module Efficiency (%) | 12.40% |
| Maximum System Voltage | 1000VDC |
| Output Terminal | Cable with Tyco Connectors |
| Weight | 11.9Kgs / 26.24lbs |
| Dimensions (L x W x H) | 1019 x 989 x 39 mm (40.12” x 38.94” x 1.54”) |
| Number of Cells | 36 in a 6 x 6 matrix connected in series |
| Solar Cell Type | Multi Crystalline Silicon |
| Solar Cell Size | 6” / 156 x 156 mm |
| Temperature Coeff. of Voc | -0.33 %/K |
| Temperature Coeff. of Isc | 0.05 %/K |
| Temperature Coeff. of Pmax | -0.43 %/K |

STC: irradiance level 1000W/m2, spectrum AM 1.5 and Cell temperature 25℃