IOP-OSPMC-135228201

Multi Crystalline Solar Modules

135W / 22.00V / 8.21A

| Specification |  |
| --- | --- |
| Maximum Power Rating (W) ± 3% | 135W |
| Open Circuit Voltage (Voc) | 22.00V |
| Short Circuit Current (Isc) | 8.21A |
| Maximum Power Voltage (Vmp) | 17.66V |
| Maximum Power Current (Imp) | 7.65A |
| Module Efficiency (%) | 13.40% |
| Maximum System Voltage | 1000VDC |
| Output Terminal | Cable with Tyco Connectors |
| Weight | 11.9Kgs / 26.24lbs |
| Dimensions (L x W x H) | 1019 x 989 x 39 mm (40.12” x 38.94” x 1.54”) |
| Number of Cells | 36 in a 6 x 6 matrix connected in series |
| Solar Cell Type | Multi Crystalline Silicon |
| Solar Cell Size | 6” / 156 x 156 mm |
| Temperature Coeff. of Voc | -0.33 %/K |
| Temperature Coeff. of Isc | 0.05 %/K |
| Temperature Coeff. of Pmax | -0.43 %/K |

STC: irradiance level 1000W/m2, spectrum AM 1.5 and Cell temperature 25℃